AW: Frequenzweiche TDL Monitor Compact

Hallo Herr Römmer-Collmann,

die Schaltungen aus Elektor und K+T würde ich gerne einsehen, dann kann ich Ihnen den Unterschied vielleicht erläutern. Meine Schaltung entspricht der Version die

in Stereoplay 11/1988 Testsieger geworden ist. Danach wurde die Weiche erst wieder in 1992 geändert. Mitteltöner ab 1992 war der 130NS 11 aus der original MONITOR von TDL.

Wichtig: die Spule L1 sollte so niederohmig sein wie möglich. Wir haben Trafokernspulen von Mundorf verwendet. Der erste Wert war 8.0mH. Irgendwann kam ein Testbericht

in der Audio mit dem Fazit, der Grundtonbereich könnte etwas wärmer sein. Daraufhin sagten viele Kunden "der Grundtonbereich könnte etwas wärmer sein". Mit einem L1= 7.0mH

wurde der Grundtonbereich dann etwas wärmer gemacht und alle waren zufrieden Habe leider keine Verstärkerempfehlung. Ich habe immer Brinkmann Stereo genommen. Habe ich heute noch für Passivboxen.

Mit freundlichen Grüßen

Axel Oberhage

----------------------

Re: TDL Monitor Compact crossover

Dear Mr Römmer-Collmann,

I would like to take a look at the circuits from Elektor and K+T, then I might be able to explain the difference to you.

My circuit corresponds to the version that won the test in Stereoplay 11/1988. The crossover was not changed again until 1992. From 1992 onwards, the midrange driver was the 130NS 11 from the original MONITOR by TDL.

Important: the L1 coil should have as low an impedance as possible. We used transformer core coils from Mundorf. The first value was 8.0mH. At some point, a test report appeared in Audio magazine with the conclusion that the fundamental range could be a little warmer.

As a result, many customers said ‘the fundamental range could be a little warmer’. With an L1=7.0mH, the fundamental range was then made a little warmer and everyone was satisfied. Unfortunately, I don't have any amplifier recommendations.

I always used Brinkmann Stereo. I still have it today for passive speakers.

Kind regards

Axel Oberhage

----------------------

From: Axel Oberhage A.O.S. Audio Systeme <info@aos-lautsprecher.de>

Sent: 28-01-2006

Subject: TDL

Dear Sir,

I have seen your homepage about TDL speakers. Let me give you a hint about our Transmission Line Speakers (DIY) in our homepage www.aos-lautsprecher.de

We were distributor from IMF TL speakers until IMF went out of the market.

Later on when TDL came to replace IMF we were distributor of TDL until TDL went out of business. While TDL started in 1993-1994 to make cheaper speakers, we started to produce TDL systems with High-End midrange and tweeters from Scan Speak / Elac.

In my Homepage I have some information about TDL/John Wright and our common work.

The Studio 4 exemption was a speaker which we had developed in Germany as a kit called MONITOR Compact. We sell this kit far more than 1200 pairs with TDL drive units but we used a better crossover. TDL thought t have a same success in creating the Studio 4 but they made some faults. The cabinets were built too cheap and the midrange should be place above the tweeter. I have supported TDL with a different crossover design and fitting better to the tweeter on top design.

Kind regards from a Transmission Line designer in Germany

Axel Oberhage